Cover

smartPX

6-Axis Drive

Quick Start Guide

ORIGINAL DOCUMENT

Manual Revision: 1.2

Software Version: 3.7.5

Firmware Version: 3.0.1.8

Revision History

| Doc. Rev. | Date | Remarks |
| --- | --- | --- |
| 1.2 | 12 Feb. 2024 | Minor corrections in Warnings and Faults chapter |
| 1.1 | 31 Jan. 2024 | Added Warnings, Faults, Inhibits. |
| 1.0 | 1 Nov. 2023 | Preliminary release. |

Copyright

© 2024 Servotronix Motion Control Ltd.

All rights reserved. No part of this work may be reproduced or transmitted in any form or by any means without prior written permission of Servotronix Motion Control Ltd.

Disclaimer

This product documentation was accurate and reliable at the time of its release. Servotronix Motion Control Ltd. reserves the right to change the specifications of the product described in this manual without notice at any time.

Trademarks

smartPX, ServoStudio+ and sensAR are trademarks of Servotronix Motion Control Ltd.

CANopen and CiA are registered trademarks of the CAN in Automation User's Group.

EtherCAT is a registered trademark of Beckhoff Automation GmbH.
Windows is a registered trademark of Microsoft Corporation.

Contact Information

Servotronix Motion Control Ltd.
21C Yagia Kapayim Street
Petach Tikva 49130 Israel
Tel: +972 (3) 927 3800
Fax: +972 (3) 922 8075
Website: www.servotronix.com

Technical Support

If you need assistance with the installation and configuration of the product, contact Servotronix technical support: tech.support@servotronix.com

Contents

1	Introduction

1.1	smartPX Product Description	5

1.2	Ordering Information	6

1.3	Product Package	7

2	Safety and Standards

2.1	Safety Symbols	8

2.2	Safety Guidelines	8

2.2.1	Installation Safety	9

2.2.2	Operational Safety	9

2.2.3	Maintenance Safety	10

3	Specifications

3.1	Dimensions	11

3.2	Electrical and Mechanical Specifications	12

3.3	Control Specifications	13

3.4	Environmental Specifications	14

3.5	STO/Functional Safety Specifications	14

3.6	Communication Specifications	14

3.7	I/O Specifications	15

3.8	Motor Feedback Specifications	16

4	Installation

4.1	Installation Overview	17

4.2	System Wiring Guidelines	18

4.2.1	Grounding	19

4.2.2	Shielding and Bonding	20

4.2.3	Input Power Filtering	20

4.3	Regeneration	20

4.4	Mounting	21

5	Interfaces

5.1	Control Board	23

5.1.1	Interface C4 – Digital Inputs and Digital Outputs	23

5.1.2	Interface C5 – Motor Brake	24

5.1.3	Interface C6 – Encoder (Motor Feedback)	25

5.1.4	Interface C7 – Safety I/Os – not yet implemented	26

5.1.5	Interfaces C1, C2, C3 – Fieldbus Communication	27

5.2	Power Board	28

5.2.1	Interface P1 – STO	28

5.2.2	Interface P2 – Power	29

5.2.3	Interfaces X1 – X6 – Motor Power	30

5.2.4	Interface C8 – USB	31

5.3	Status LEDs	31

5.4	Power Up	32

6	Interface Circuitry

6.1	Digital Inputs Circuitry	33

6.2	Digital Output Circuitry	34

6.3	Motor Brake Circuitry	35

7	ServoStudio+

7.1	Software Installation	36

7.2	Activation	36

7.3	Parameter Values	38

8	Drive Configuration

8.1	Motors	39

8.1.1	Load Motor Parameters	39

8.1.2	Modify Motor Parameters	40

8.1.3	Copy Parameter Values to Another Motor	41

8.1.4	Save Motor Parameters	41

8.2	Feedback	42

8.3	Limits	44

8.4	Digital I/Os	45

8.5	Autotune	46

8.6	Control Loop	48

8.7	Upgrade Drive Firmware	51

9	Warnings and Faults

9.1	Warning Codes	52

9.2	Fault Codes	55

9.3	Inhibit Codes	64

# Introduction

## smartPX Product Description

The smartPX is a six-axis servo drive developed by Servotronix that supports rotary motors with brakes. The drive is primarily, but not exclusively, intended for use in robot applications.

Figure ‎1-1.  smartPX 6-Axis Servo Drive

## Ordering Information

|  |  | PX | – | 6 | – | A | – | 2A | – | EC | – | RO | – | ST | – | M | – | NN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | smartPX multi-axis servo drive series |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Number of Axes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Supports 6 axes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Supports 4 axes * |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A | Axis; Rated Arms; Max Arms. 1; 4.5; 18.1. 2; 4.5; 18.1. 3; 3; 13.6. 4; 2; 9.1. 5; 2; 9.1. 6; 2; 9.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Voltage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2A | 1-phase 220 VAC, -15%/+20%, 50/60 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4D | 3-phase 220 VAC, -15%/+20%, 50/60 Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Communication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EC | EtherCAT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PN | Profinet * |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CN | CANopen * |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Motor Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R | Supports rotary motor only |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L | Supports linear motor/rotary motor * |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Safety |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ST | STO function only |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SA | Safety functions – STO/SS1/SBC * |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Feedback Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M | Supports sensAR/Tamagawa encoders |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Controller Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MC | softMC * |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CS | CodeSys * |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NN | Not installed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* In development. Feature will be supported in the future.

## Product Package

The smartPX package includes:

smartPX multi-axis servo drive

STO bypass plug

Mating connectors, as specified in the following table. For more details, refer to Chapter 5: Interfaces.

Table ‎1-1.	Mating Connectors

| Part Description | Servotronix Part Number | Quantity |
| --- | --- | --- |
| P1 STO connector | 15EDGKNM-4P4-GSn-3.5 | 1 |
| P2 AC IN connector | 2EDGKMB-7P7-BKSn-5.08 | 1 |
| X1-X6 Motor connector | 2EDGKMB-4P4-BKSn-5.08 | 6 |
| C4 I/O connector | 15EDGKNHGB-16P16-BKSn-3.5 | 1 |
| C5 Brakes connector | 15EDGKNHGB-14P14-BKSn-3.5 | 1 |

# Safety and Standards

## Safety Symbols

Safety symbols indicate a potential for personal injury or equipment damage if the prescribed precautions and safe operating practices are not followed.

The following safety-alert symbols are used on drive and in the documentation.

| Symbol | Meaning | Description | ISO 7000/ IEC 60417 |
| --- | --- | --- | --- |
|  | Caution | Indicates caution is necessary when operating the device. Also indicates the current situation needs operator awareness or operator action to avoid undesirable consequences. | 0434 |
|  | Dangerous voltage | Indicates hazards arising from dangerous voltages. | 5036 |
|  | Protective earth; protective ground | Identifies any terminal which is intended for connection to an external conductor for protection against electric shock in case of a fault, or the terminal of a protective earth (ground) electrode. | 5019 |

## Safety Guidelines

The smartPX servo drive is intended for use as a component within a machine system.

The machine builder and integrator must ensure the protection of both personnel and the complete machine system.

The machine builder and/or integrator must perform a risk assessment in view of using the smartPX drive in the intended application. Based on the results, the appropriate safety measures must be implemented.

The smartPX drive must be used in compliance with all applicable safety regulations and directives, and all technical specifications and requirements.

|  | The smartPX drive utilizes hazardous voltages. It must be properly grounded. |
| --- | --- |

|  | The machine builder and the machine owner are responsible for the safety of the machine operators. |
| --- | --- |

|  | The machine owner and the machine operator are responsible for ensuring personnel cannot enter the hazard zone while the machine is energized unless adequate functional safety mechanisms are in place. |
| --- | --- |

Only qualified personnel may perform installation, operation, service and maintenance procedures. These persons must have sufficient technical training and knowledge to foresee and recognize potential hazards that may occur when using the product, modifying settings, and operating the mechanical, electrical and electronic components of the entire machine system.

All persons working on and with the product must be fully familiar with all applicable standards, directives, and accident prevention regulations when performing such work.

### Installation Safety

| Note | Incorrect handling of the smartPX may cause personal injury and/or damage to equipment. |
| --- | --- |

|  | When connecting the smartPX to other control equipment, be sure to follow two basic guidelines to prevent damage to the drive: The smartPX must be grounded via the earth ground of the main AC voltage supply. Any motion controller, PLC or PC that is connected to the smartPX must be grounded to the same earth ground as the smartPX. |
| --- | --- |

Before installing or commissioning the smartPX, review all relevant product documentation.

Perform the installation in strict compliance with product specifications and installation instructions.

All system components must be connected to ground. Electrical safety is provided through a low-resistance earth ground connection. (Protective Class 1 according standard EN/IEC 618005-1.) The motor should be connected to protective earth by an independent earthing conductor rated not less than the motor wire.

As part of the machine design, the machine builder must generate a hazard analysis for the machine and take appropriate measures to ensure that unforeseen movements cannot cause personal injury and/or damage to equipment.

Wiring of a yellow color with or without one or more green stripes must not be used, except for protective bonding.

Power cables should be rated at least 600V, 75℃.

### Operational Safety

| Note | Machine builders are responsible for machine safety implementation, testing and certification. The machine manual must define operational and maintenance conditions and safety precautions. |
| --- | --- |

Perform all machine operations in strict compliance with product specifications and installation instructions.

The machine builder must provide a power disconnect device in accordance with local regulations.

During operation, keep all covers and cabinet doors shut.

During operation the machine has electrically charged components and hot surfaces. Power cables can carry a high voltage, even when the motor is not rotating.

Machine axes with a suspended load or unbalanced load must have an additional mechanical safety block(such as a motor-holding brake) to prevent the load from falling out of control. The smartPX cannot keep the load suspended when STO is active. Serious injury may result if the load is not properly safeguarded.

### Maintenance Safety

| Note | Incorrect handling of the smartPX may cause personal injury and/or damage to equipment. |
| --- | --- |

Before performing maintenance on the smartPX (or the machine it drives), review all relevant product documentation.

Perform maintenance procedures in strict compliance with the product maintenance requirements and instructions.

To prevent electric arcing and hazards to personnel and electric contacts, never disconnect of connect the product while the power source is energized.

After disconnecting the power source from the machine, wait at least 5 minutes before touching or disconnecting parts of the machine that normally carry electrical charges (such as capacitors, contacts, screwed connections).

Before touching the machine, measure the electrical contact points with a meter. Be certain voltage is below 30 VDC before handling components.

# Specifications

## Dimensions

Figure ‎3-1.  Dimensions (mm)

## Electrical and Mechanical Specifications

Table ‎3-1.	Electrical and Mechanical Specifications

| Feature | Specification |  |
| --- | --- | --- |
| Input power | Input AC line voltage | 1-phase/3-phase: 220 VAC, -15%/+20% |
|  | Line Frequency | 47–63 Hz |
|  | Continuous (rated) current | 16.5 A |
| STO | Voltage | 24V, -15%/+20% |
|  | Peak (max) input current for 2 seconds | 0.38A |
| Logic supply | Voltage | 220 VAC, -15%/+20% |
|  | Peak (max) input current for 2 seconds | 0.18A |
| Drive output | Axis 1 – Continuous (rated) current | 4.5 Arms |
|  | Axis 1 – Peak (max) current for 2 seconds | 18.1 Arms |
|  | Axis 2 – Continuous (rated) current | 4.5 Arms |
|  | Axis 2 – Peak (max) current for 2 seconds | 18.1 Arms |
|  | Axis 3 – Continuous (rated) current | 3 Arms |
|  | Axis 3 – Peak (max) current for 2 seconds | 13.6 Arms |
|  | Axis 4 – Continuous (rated) current | 2 Arms |
|  | Axis 4 – Peak (max) current for 2 seconds | 9.1 Arms |
|  | Axis 5 – Continuous (rated) current | 2 Arms |
|  | Axis 5 – Peak (max) current for 2 seconds | 9.1 Arms |
|  | Axis 6 – Continuous (rated) current | 2 Arms |
|  | Axis 6 – Peak (max) current for 2 seconds | 9.1 Arms |
|  | PWM frequency | 16 kHz |
| External regen resistor | Operation DC bus voltage | 400 VDC |
|  | Minimum resistance | 30 Ω |

## Control Specifications

Table ‎3-2.	Control Specifications

| Feature | Specification |  |
| --- | --- | --- |
| Motors | Types | Rotary servo |
|  | Motor Phasing | Automatic configuration of motor phasing, phase-swapping |
|  | Motor Direction | Clockwise/counter-clockwise, encoder direction |
| Operation Modes | Control Modes | Current control, Velocity control, Position control, Continuous synchronous position, Continuous synchronous torque |
| Current (Torque) Control | Update Rate | 62.5 μs (16 kHz) |
|  | Control Loop | PI, Feedforward |
|  | Autotuning | Automatic setting of current control loop parameters |
| Velocity Control | Update Rate | 125 μs (8 kHz) |
|  | Control Loop | PDFF, Feedforward |
|  | Filters | First order low pass filter, Double first order low pass filter, Notch filters, High pass filter, Band pass filter, User defined polynomial filter |
| Position Control | Update Rate | 125 μs (8 kHz), |
|  | Control Loop | PID controller, |
| Reference Command | Protocol | CANopen over EtherCAT (CoE) |
| Advanced Control Functions |  | Velocity observer, Friction compensation, Harmonic compensation, Collision detection, Input shaping, Gain scheduling |
| Tuning | Methods | Autotuning, Frequency analyzer, Inertia and friction estimation |
| Rotary Units | Position | Revolutions, counts, degrees |
|  | Velocity | rps, rpm, deg/s |
|  | Acc/Dec | rps/s, rpm/s, deg/s2 |
| User Interface | GUI | ServoStudio+ |
|  | Functions | Configuration, Tuning, Monitoring, Analysis |

## Environmental Specifications

Table ‎3-3.	Environmental Specifications

| Feature | Specification |
| --- | --- |
| Protection class | IP20, pollution degree: 2 as per IEC 60664-1 Do not use in the following locations: corrosive or flammable gasses, water oil or chemical, dust including iron dust and salts |
| Ambient temperature | 0° to 45°C no derating |
| Storage temperature | 0° to 70°C |
| Humidity | 10 to 90% |
| Altitude | <1000m no derating |
| Vibration | 1.0g |

## STO/Functional Safety Specifications

| Note | STO certification pending. |
| --- | --- |

Table ‎3-4.	STO/Functional Safety Specifications

| Feature | Specification |
| --- | --- |
| Nominal voltage | 24 VDC |
| Voltage levels | 15–30 VDC STO function not active (motion allowed) 0–5 VDC STO function active (motion not allowed) 5–15 VDC STO function not defined and not guaranteed |
| Power supply – external | SELV/PELV required |
| Power supply – internal | STO jumper required |
| Maximum reaction time | 30 ms |

## Communication Specifications

Table ‎3-5.	Communication Specifications

| Feature | Specification |  |
| --- | --- | --- |
| Fieldbus | Type | EtherCAT over Ethernet (CoE), 100 Mbit |
|  | Communication cycle time (minimum) | 500 µs |
|  | Device configuration file | IEC61800-7 Profile type1 (CiA402) CoE (CANopen over EtherCAT) |
|  | Communication object | PDO (Process Data Object) SDO (Service Data Object) |
|  | Synchronization type | SYNC0 Event Synchronization |

## I/O Specifications

Table ‎3-6.	STO/Functional Safety Specifications

| Feature | Specification |  |
| --- | --- | --- |
| Digital inputs | Quantity | 4 |
|  | Signal | Opto-isolated. All 4 digital inputs must be wired/used either as sink or as source. |
|  | Voltage | 24V – External, user-supplied On: 11 to 30 VDC Off: 0 to 5 VDC |
|  | Max. input current | 12.5 mA per input |
|  | Propagation delay time | 1 ms |
| Fast digital inputs | Quantity | 4 |
|  | Signal | Opto-isolated. All 4 digital inputs must be wired/used either as sink or as source. |
|  | Voltage | 24V – External, user-supplied On: 11 to 30 VDC Off: 0 to 5 VDC |
|  | Max. input current | 12.5 mA per input |
|  | Propagation delay time | 1 μs |
| Digital outputs | Quantity | 6 |
|  | Signal | Opto-isolated. All 6 digital outputs must be wired/used either as sink or as source. |
|  | Voltage | 24 V |
|  | Max. output current | 50 mA |
|  | Propagation delay time | 1 ms |
| Motor brakes | Quantity | 6, independently controlled outputs |
|  | Voltage | 24V |
|  | Maximum current per axis | 1A |
|  | Maximum current total | 6A |

## Motor Feedback Specifications

Table ‎3-7.	Motor Feedback Specifications

| Feature | Specification |
| --- | --- |
| Supply voltage from drive | 5.15 VDC ±2% |
| Max. supply current from drive | 720 mA per axis |
| Recommended cable type | Shielded twisted-pair copper wire |
| Encoder type | Tamagawa MT absolute encoder, multi-turn 16-bit/single-turn 17-bit/23-bit, 2.5 Mbps |
|  | Tamagawa ST absolute encoder, single-turn 17-bit/23-bit, 2.5 Mbps |
|  | Servotronix sensAR absolute encoder, multi-turn 16-bit/single-turn 20-bit, 2.5 Mbps |

# Installation

## Installation Overview

Perform the following steps to install and setup a smartPX system.

- Mount the smartPX.

Make all wiring and cable connections, as required by your application:

I/Os

Motor feedback devices (encoders)

Fieldbus devices, if in use

Safe torque off (STO), or bypass using jumpers

Motors

Regeneration resistor, if required

Motor brakes, if required

AC and DC input voltage

Fans

Safety relay, if required

Connect the drive to the user computer.

Power up the drive and the user computer.

Install ServoStudio+ software.

## System Wiring Guidelines

|  | When connecting the smartPX to other control equipment, be sure to follow two basic guidelines to prevent damage to the drive: The smartPX must be grounded via the earth ground of the main AC voltage supply. Any motion controller, PLC or PC that is connected to the smartPX must be grounded to the same earth ground as the smartPX. |
| --- | --- |

Figure ‎4-1. High voltage wiring

Figure ‎4-2.  Emergency stop wiring

Figure ‎4-3.  Motors wiring

### Grounding

System grounding is essential for proper performance of the drive system.

The AC input voltage ground wire must be connected to a PE terminal on the drive. This is necessary for both safety and EMI reduction.

Use a single point ground for the system (star wiring) to avoid ground loops.

It is strongly recommended that the smartPX be mounted to a metallic back panel, and that a high frequency ground be provided to connect the back panel to earth ground. Provide an electrical connection across the entire back surface of the drive panel. Electrically-conductive panels such as aluminum or galvanized steel are recommended. For painted and other coated metal panels, remove all coating behind the drive. The objective is to provide an extremely low impedance path between the filters, drives, power supplies, and earth ground for high-frequency signals that might cause EMI. Use a flat braid or copper busbar to achieve high-frequency grounding. Use the shortest braid possible when connecting high frequency grounds.

Ensure good connections between the cabinet components. Connect the back panel and cabinet door to the cabinet body using several conductive braids. Never rely on hinges or mounting bolts for ground connections. Ensure good ground connection from cabinet to proper earth ground. Ground leads should be the same gauge as the leads to main power or one gauge smaller.

The user computer must also be properly grounded.

### Shielding and Bonding

To minimize noise emissions and maximize the immunity levels of the drive system, motor and feedback cables must be shielded and properly bonded to a grounded surface.

The shield must be connected to ground at both ends of the cable. Its effect is to reduce the impedance between the cable shield and the back panel.

It is recommended that all shielded cables be bonded to the back panel.

The motor and feedback shielded cables should be exposed as close as possible to the drive. This exposed cable shield is bonded to the back panel using either non-insulated metallic cable clamps or cable bonding clamps.

It is recommended to use a star point shield connection with a shielding busbar.

### Input Power Filtering

The drive’s electronic system components require EMI AC line filtering in the input power leads to meet the CE requirements for the industrial environment.

Care must be taken to adequately size the system. The type of filter is determined according to the voltage and current rating of the system and whether the incoming line is single phase or three phase. One input line filter can be used for multi-axis control applications.

Implementation of the input power filter must adhere to the following guidelines:

Maintain separation of leads entering and exiting the mains filter.

Filter must be mounted on the same panel as the drive.

Filter must be mounted as close as possible to the drive, to prevent noise from being capacitively coupled into other signal leads and cables.

When mounting the filter to the panel, remove any paint or material covering. Use an unpainted metallic back panel, if possible.

Filters are provided with a ground terminal, which must be connected to ground.

Filters can produce high leakage currents. Filters must be grounded before connecting the supply.

Filters should not be touched for 10 seconds after removing the supply.

## Regeneration

When a motor and load decelerate rapidly, kinetic energy of the load drives the motor shaft energy back to the drive. This regeneration energy must be dissipated or absorbed.

Regenerative energy that is not dissipated is added to the electrical energy already stored in the capacitors of bus module. If the capacitors can absorb the energy, the system does not require a regeneration resistor. However, if excessive energy is added, the voltage on the capacitors will exceed the over-voltage threshold, which then disables the drive.

To prevent over-voltage, and potential damage to the system, excessive regenerative energy must be dissipated by a regeneration resistor.

If your application requires a regenerative resistor for braking a high inertia load during emergency stops, connect an external regenerative resistor to interface P2, terminals B and P.

## Mounting

The smartPX can be mounted in either a horizontal or vertical position.

In addition, the drive can be mounted entirely within a cabinet, or with the heatsink protruding through the back of the cabinet.

It is important to maintain an ambient temperature that does not exceed 45°C. Therefore, two cooling fans must also be installed near the smartPX on the side of the main Power interface (P2). The recommended distance is 30 mm.

The distance between the two fans, the smartPX, the cabinet and/or other objects must also be at least 30 mm.

Figure ‎4-4.  Mounting dimensions

# Interfaces

All electrical interfaces are on the top panel of the smartPX.

Figure ‎5-1. SmartPX Electrical Interfaces

## Control Board

### Interface C4 – Digital Inputs and Digital Outputs

Figure ‎5-2. 	Interface C4

All digital inputs and digital outputs on all smartPX models are opto-isolated.

Wire the digital inputs and outputs according to the requirements of your application. Unused pins must remain unwired.

| Note | All 8 digital inputs must be wired/used either as sink or as source. All 6 digital outputs must wired/used either as sink or as source. |
| --- | --- |

Table ‎5-1.	Interface C4 Pinout

| Pin | Label | Description | Pin | Label | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | IN 1 | Opto-isolated digital input 1. | 2 | OUT 1 | Opto-isolated digital output 1. |
| 3 | IN 2 | Opto-isolated digital input 2. | 4 | OUT 2 | Opto-isolated digital output 2. |
| 5 | IN 3 | Opto-isolated digital input 3. | 6 | OUT 3 | Opto-isolated digital output 3. |
| 7 | IN 4 | Opto-isolated digital input 4. | 8 | OUT 4 | Opto-isolated digital output 4. |
| 9 | IN 5 | Opto-isolated digital input 5. Supports 500 kHz fast input. | 10 | OUT 5 | Opto-isolated digital output 5. |
| 11 | IN 6 | Opto-isolated digital input 6. Supports 500 kHz fast input. | 12 | OUT 6 | Opto-isolated digital output 6. |
| 13 | IN 7 | Opto-isolated digital input 7. Supports 500 kHz fast input. | 14 | COM OUT | Common output |
| 15 | IN 8 | Opto-isolated digital input 8. Supports 500 kHz fast input. | 16 | COM IN | Common input |

Table ‎5-2. 	Interface C4 Mating Connector

| Connector | Pluggable terminal block, pitch 3.5 mm, 2*8 |
| --- | --- |
| Mating connector | DEGSON 15EDGKNHG-3.5-16P-13-28Z(H) |

### Interface C5 – Motor Brake

Figure ‎5-3.	Interface C5

Table ‎5-3. 	Interface C5 Pinout

| Pin | Label | Description | Pin | Label | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | BR 1 + | Motor 1 brake output + | 2 | BR 1 – | Motor 1 brake output - |
| 3 | BR 2 + | Motor 2 brake output + | 4 | BR 2 – | Motor 2 brake output - |
| 5 | BR 3 + | Motor 3 brake output + | 6 | BR 3 – | Motor 3 brake output - |
| 7 | BR 4 + | Motor 4 brake output + | 8 | BR 4 – | Motor 4 brake output - |
| 9 | BR 5 + | Motor 5 brake output + | 10 | BR 5 – | Motor 5 brake output - |
| 11 | BR 6 + | Motor 6 brake output + | 12 | BR 6 – | Motor 6 brake output - |
| 13 | 24 V + | Motor brake power 24 VDC | 14 | 24 V – | Motor brake power 24 VDC return |

Table ‎5-4. Interface C5 Mating Connector

| Connector | Pluggable terminal block, pitch 3.5 mm, 2*8 |
| --- | --- |
| Mating connector | DEGSON 15EDGKNHG-3.5-14P-13-28Z(H) |

### Interface C6 – Encoder (Motor Feedback)

Figure ‎5-4.	Interface C6

Wire the digital inputs and outputs according to the requirements of your application. Unused pins must remain unwired.

Table ‎5-5. 	Interface C6 Pinout

| Pin | Function | Description |
| --- | --- | --- |
| 1 |  | Reserved. Not in use. |
| 2 |  | Reserved. Not in use. |
| 3 |  | Reserved. Not in use. |
| 4 |  | Reserved. Not in use. |
| 5 |  | Reserved. Not in use. |
| 6 |  | Reserved. Not in use. |
| 7 | GND_6 | Axis 6 Ground (5V return) |
| 8 | ENC_Dn_Ax6 | Axis 6 Serial encoder data - |
| 9 | GND | Axis 5 Ground (5V return) |
| 10 | ENC_Dn_Ax5 | Axis 5 Serial encoder data - |
| 11 | GND | Axis 4 Ground (5V return) |
| 12 | ENC_Dn_Ax4 | Axis 4 Serial encoder data - |
| 13 | GND | Axis 3 Ground (5V return) |
| 14 | ENC_Dn_Ax3 | Axis 3 Serial encoder data - |
| 15 | GND | Axis 2 Ground (5V return) |
| 16 | ENC_Dn_Ax2 | Axis 2 Serial encoder data - |
| 17 | GND | Axis 1 Ground (5V return) |
| 18 | ENC_Dn_Ax1 | Axis 1 Serial encoder data - |
| 19 |  | Reserved. Not in use. |
| 20 |  | Reserved. Not in use. |
| 21 |  | Reserved. Not in use. |
| 22 |  | Reserved. Not in use. |
| 23 |  | Reserved. Not in use. |
| 24 |  | Reserved. Not in use. |
| 25 | Enc_5V_Ax6 | Axis 6 Power supply 5V |
| 26 | ENC_Dp_Ax6 | Axis 6 Serial encoder data + |
| 27 | Enc_5V_Ax5 | Axis 5 Power supply 5V |
| 28 | ENC_Dp_Ax5 | Axis 5 Serial encoder data + |
| 29 | Enc_5V_Ax4 | Axis 4 Power supply 5V |
| 30 | ENC_Dp_Ax4 | Axis 4 Serial encoder data + |
| 31 | Enc_5V_Ax3 | Axis 3 Power supply 5V |
| 32 | ENC_Dp_Ax3 | Axis 3 Serial encoder data + |
| 33 | Enc_5V_Ax2 | Axis 2 Power supply 5V |
| 34 | ENC_Dp_Ax2 | Axis 2 Serial encoder data + |
| 35 | Enc_5V_Ax1 | Axis 1 Power supply 5V |
| 36 | ENC_Dp_Ax1 | Axis 1 Serial encoder data + |

Table ‎5-6. Interface C6 Mating Connector

| Mating connector (shell) | 3M 10336-52F0-008 D-subminiature connector 36P junction shell SLD |
| --- | --- |
| Mating connector (plug) | 3M 10136-3000PE D-subminiature connector 36P MDR plug |

### Interface C7 – Safety I/Os – not yet implemented

Figure ‎5-5. 	Interface C7

### Interfaces C1, C2, C3 – Fieldbus Communication

Interfaces C1, C2 and C3 are RJ45 ports that serve as transmitter and receiver for a drive operating on a EtherCAT network.

Figure ‎5-6. 	Interfaces C1 C2 C3

Table ‎5-7. 	Interfaces C1 C2 C3 Pinout

| Pin | Name | Description |
| --- | --- | --- |
| 1 | TX+_D1 | Transmit data + |
| 2 | TX-_D1 | Transmit data - |
| 3 | RX+_D2 | Receive data + |
| 4 |  |  |
| 5 |  |  |
| 6 | RX-_D2 | Receive data - |
| 7 |  |  |
| 8 |  |  |

Table ‎5-8.	Interfaces C1 C2 C3 Mating Connector

| Mating connector | Any RJ45 CAT6 compatible shielded connector |
| --- | --- |
| Cable | CAT6 or higher, standard Ethernet straight cable |

## Power Board

|  | Make sure the main voltage rating matches the drive specification. Applying incorrect voltage may cause drive failure. Do not apply power until all hardware connections are complete. |
| --- | --- |

### Interface P1 – STO

Figure ‎5-7. 	Interface P1

Table ‎5-9. 	Interface P1 Pinout

| Pin | Label | Function |
| --- | --- | --- |
| 1 | 24V | STO Power Supply (STO Enable) |
| 2 | GND | STO Return |
| 3 | GND | Reserved for STO. Internal return. |
| 4 | 24V | Reserved for STO. Internal supply. |

Table ‎5-10. 	Interface P1 Mating Connector

| Connector (socket) | DEGSON 15EDGVM-THR-3.5-04P-13-10Z(H) |
| --- | --- |
| Mating connector (plug) | DEGSON 15EDGKNM-3.5-04P-14-1022Z(H) |
| Wire gauge | 24–20 AWG |
| Strip length | 20 mm |

STO Enable and STO Return must be connected to enable smartPX operation. 
The STO Enable signal voltage must be 24 VDC.

| Note | If the application does not require STO control, jumper pin 4 to pin 1, and pin 3 to pin 2, to bypass the STO, using the STO bypass plug supplied with the smartPX. |
| --- | --- |

Figure ‎5-8. 	STO Bypass Plug

### Interface P2 – Power

Figure ‎5-9. 	Interface P2

Table ‎5-11. 	Interface P2 Pinout

| Pin | Label | Description |
| --- | --- | --- |
| 1 | R | AC Phase 1 (L1) (Reference) |
| 2 | S | VAC AC Phase 2 (L2) (Secondary) |
| 3 | T | VAC AC Phase 3 (L3) (Tertiary) |
| 4 | L1 | Logic AC (phase) |
| 5 | L2 | Logic AC (neutral) |
| 6 | B | Regen + (DC bus +) |
| 7 | P | Regen - |

Table ‎5-12. 	Interface P2 Mating Connector

| Connector (socket) | DEGSON 2EDGVM-5.08-07P-13-100Z(H) |
| --- | --- |
| Mating connector (plug) | DEGSON 2EDGKM-5.08-07P-13-1000A(H) |
| Wire gauge | 28–12 AWG |
| Strip length | 7 mm |

|  | Make sure the main voltage rating matches the drive specification. Applying incorrect voltage may cause drive failure. Do not apply power until all hardware connections are complete. |
| --- | --- |

|  | Prevent inrush surge: Bus Power (R-S-T): After switching off bus power, wait 1 minute before switching on again. Logic Power (L1-L2): After switching off logic power, wait 1 minute before switching on again. |
| --- | --- |

###### Procedure:	Connecting Power

Figure ‎5-10.	Ground Terminals

The smartPX has 4 ground PE terminals, located on its mounting plate/heatsink.

- Connect the AC input voltage ground wire to one of the PE terminals. Use an M4 ring or spade terminal.

Connect R, S and T (for bus power).

If the main voltage is from a single-phase source, connect line and neutral to R and T.

If the main voltage is from a three-phase source, connect the phases to R, S and T.

Connect L1 and L2 (for logic power).

If the main voltage is from a single-phase source, connect line and neutral to L1 and L2.

If the main voltage is from a 3-phase source, connect any two phases to L1 and L2.

### Interfaces X1 – X6 – Motor Power

Figure ‎5-11. 	Interfaces X1 – X6

Table ‎5-13. 	Interfaces X1 – X6 Pinout

| Pin | Label | Description |
| --- | --- | --- |
| 1 | PE | Protective ground |
| 2 | U | Motor phase U |
| 3 | V | Motor phase V |
| 4 | W | Motor phase W |

Table ‎5-14. 	Interfaces X1 – X6 Mating Connector

| Connector (socket) | DEGSON 2EDGVM-5.08-04P-13-100Z(H) |
| --- | --- |
| Mating connector (plug) | DEGSON 2EDGKM-5.08-04P-13-1000A(H) |
| Wire gauge | 28–12 AWG |
| Strip length | 7 mm |

### Interface C8 – USB

Figure ‎5-12. 	Interface C8

C8 is a USB type C port that enables save and restore of program and data files from/to the smartPX.

To backup and restore files, you must use the ServoStudio+ software.

## Status LEDs

Figure ‎5-13. 	Status LEDs

#### State LED

Table ‎5-15.	State LED

| Color/Behavior | Meaning |
| --- | --- |
| Red | Fault (including “inhibit”) |
| Green | All axes are enabled |
| Blinking green | Some of the axes are enabled |
| Blue | Drive is operational, but no axis is enabled |
| Blinking blue | E-stop inputs are activated |

#### Axis LEDs

Table ‎5-16.	Axis Status LED

| Color/Behavior | Meaning |
| --- | --- |
| Green | Axis is enabled |
| Red | Axis is disabled |
| Blinking red | Axis has fault |

#### Vbus LED

Figure ‎5-14. 	Voltage LED

The Vbus LED provides a high voltage warning.

Do not touch or attempt to disconnect any cables until the LED shuts off.

Table ‎5-17.	Vbus LED

| Color/Behavior | Meaning |
| --- | --- |
| Red On | Vbus higher than 60V |
| Red Off | Vbus less than 60V |

#### Brake LEDs

Figure ‎5-15. 	Brake LEDs

Table ‎5-18.	Brake LEDs

| Color/Behavior | Meaning |
| --- | --- |
| Green On | Brake released. |
| Green Off | Brake engaged. |

## Power Up

After completing all hardware connections, you can turn on power to the drive.

Check the status LEDs after the drive is powered on.

# Interface Circuitry

## Digital Inputs Circuitry

| Note | Input ON range: 11–30 VDC Input OFF range: 0–5 VDC |
| --- | --- |

Figure ‎6-1. Digital inputs circuitry

## Digital Output Circuitry

Figure ‎6-2. 	Digital outputs circuitry

## Motor Brake Circuitry

Figure ‎6-3. 	Motor brake circuity

# ServoStudio+

ServoStudio+ is web-based software for setting motor parameters and configuring the smartPX drive.

## Software Installation

ServoStudio+ is compatible with Windows 10 and later operating systems. If you require assistance with an older version of Windows, contact Servotronix Technical Support.

Download the ServoStudio+ software installation file (ServoStudio+_3.7.x.MSI) from the Servotronix website [LINK], or contact Technical Support.

Run the file.

## Activation

- Activate ServoStudio+ from the desktop icon or the Start menu.

Log in to the system:

Default username: admin

Default password: ADMIN

It is recommended that you change these credentials after logging in for the first time. User names and passwords can be modified in Settings > User Manager.

Figure ‎7-1. 	Login

To activate ServoStudio+ from your desktop or tablet, you need the IP address of your smartPX. The drive is factory-configured with default address: 192.168.57.100.
Contact your IT manager to ensure that your PC is on the same network.

Enter the drive IP address, and click Connect.

| Note | Some ServoStudio+ functions are available without a connection to an actual drive. If you select Use in Offline Mode, the software will start without requiring login authentication. |
| --- | --- |

ServoStudio+ opens to the home screen.

Axes should be disabled during the basic configuration. If an axis icon is lit (green), it indicates the axis is enabled.

To disable an axis, do either of the following:

Click the enable/disable button (the POWER icon) in the toolbar at the top of the ServoStudio+ window.

Click the axis number in the toolbar. Then use the axis toggle button in the axes pane.

| Axis color | Meaning |
| --- | --- |
| White | Axis disabled |
| Green | Axis enabled |
| Yellow | Axis disabled with a warning |
| Red | Axis disabled with a fault |
| Green/Yellow split | Axis enabled, with a warning |

Figure ‎7-2. 	Home screen

Open the ServoStudio+ navigation menu.

Figure ‎7-3. 	Navigation menu

## Parameter Values

The first time a new drive is connected to ServoStudio+, the software displays the drive’s factory-defined (default) parameter values.

ServoStudio+ reads and displays values from the drive RAM.

After you edit a parameter value, press Enter, or press Tab to move the next field. The new value is sent to the drive RAM, and an update message is displayed.

To save parameter values to the drive’s non-volatile memory at any time, press the Save button on the ServoStudio+ toolbar. If not saved, modified parameter values are lost upon power cycle.

To save and load parameter values to and from PC files, go to ServoStudio+ Settings > File Manager.

# Drive Configuration

This section describes the basic steps for configuring the drive system using the ServoStudio+ interface.

## Motors

The Motor screen allows you to select a motor from ServoStudio+ databases. When you select the motor family and model number, ServoStudio+ sets the appropriate motor and feedback parameters.

ServoStudio+ contains two databases of motor settings.

The ServoStudio+ Library contains a database of motors already defined and supported by Servotronix. These motor settings cannot be modified by users.

The User Library contains all other motors defined by users.

### Load Motor Parameters

To load a set of motor parameters:

- In ServoStudio+, select Configuration > Motors.

Figure ‎8-1. 	Motors

| Choose from Database | Opens a menu that lets you select a motor from either the ServoStudio+ library or the User library. |
| --- | --- |
| Import from File | Prompts you to load motor parameters to ServoStudio+ from a JSON file. |

Select an axis from the menu bar at the top of the ServoStudio+ screen.

Select a motor from one of the ServoStudio+ motor libraries.

Select Choose from Database.

Select a motor from the Family and Model lists.

Alternately, select a motor parameter file.

- Select Import from File.

- Select the JSON file containing the required motor parameters.

ServoStudio+ loads and displays the appropriate motor and feedback parameters.

### Modify Motor Parameters

To modify parameter values.

- Click in a parameter field, and enter a new value.

The modified field is highlighted, and you are prompted to save or discard the changes.

Figure ‎8-2. 	Save changes to axis (saves parameter in drive)

Select Save Changes to Axis n.

Save Changes writes the new parameter values to the drive, but does not save the parameter values to the database or a file.

### Copy Parameter Values to Another Motor

To copy parameter values from one motor to another:

- With modified parameter fields highlighted, select the number of the axis you want to copy to.

Select Save Changes to Axis.

### Save Motor Parameters

| Save to Database | Saves motor parameter settings to the database. Does not write to the drive. New. When modifying parameter values of a motor in the ServoStudio+ motor library, you can save the changes only by defining a new motor in the User library. Replace Existing. You can overwrite a file in the User library. |
| --- | --- |
| Export to File | Prompts you to save motor parameters from ServoStudio+ to a JSON file. |

## Feedback

- In ServoStudio+, select Configuration > Feedback.

- Open the General tab.

Figure ‎8-3. 	Feedback

From the Feedback Type list, select the motor encoder.

Modify other parameters as required by your application.

| Feedback Type | The motor feedback device being used in the application |
| --- | --- |
| Position Offset | An offset value that is added to the internal cumulative position counter, to give the position feedback value. |
| Position Feedback | The position feedback value. Read only. |
| Reverse motor direction | When switched on, the values of position feedback, velocity and current are inverted, thereby reversing the direction of motor movement. |

Advanced options are not required for basic configuration.

Open the Phase Find tab.

Phase find is a procedure that initializes commutation for encoder systems.

Figure ‎8-4. 	Phase find

ServoStudio+ for smartPX uses the zeroing process for determining the commutation offset on motors with an absolute encoder. Zeroing locks the rotor in place by passing a fixed current through two phases.

Confirm or modify the value of Phase Find Max Current Amplitude.

| Phase Find Max Current Amplitude | The fixed current used for the zeroing procedure. |
| --- | --- |

Select Start Zeroing Process.

When zeroing is activated, the drive rotates and locks the motor at the fixed electrical angle with a fixed current. Locking occurs after stopping briefly at a temporary location to overcome static friction, or after starting at an electrical angle 180° away from the final locked position.

The number of motor electrical cycles per one mechanical revolution is equal to the number of pairs of magnets. Each pair of magnets in the motor creates a 0—360 degree electrical cycle. Therefore, if the motor has 8 individual magnetic poles, the motor can be locked at four different places in one mechanical revolution.

When zeroing is complete, a new motor phase is found. ServoStudio+ prompts you to save the new value.

Repeat for each axis.

## Limits

- In ServoStudio+, select Configuration > Limits.

Figure ‎8-5. 	Limits

Modify the values for position, velocity and current limits as required by your application.

| Position Limits |  |
| --- | --- |
| Position Error Max | The maximum position error value that does not produce a fault. |

| Velocity Limits |  |
| --- | --- |
| Motor Speed | The maximum motor speed, according to the motor datasheet. |
| Velocity Error Max | The maximum velocity error value that does not produce a fault. |
| Velocity Limit | The maximum (user-defined) velocity limit. |
| Velocity Limit Actual | The actual velocity limit for the application, which is the lowest (minimum) value of all velocity limit inputs, such as motor speed and velocity limit (user limit). |

| Current |  |
| --- | --- |
| Drive Peak (Max) | The peak (max) current of the drive axis. |
| Motor Peak (Max) | The peak (max) current of the motor, according to the motor datasheet. |
| Current Limit | The maximum (user-defined) current limit |
| Current Limit Actual | The actual current limit for the application, which is the lowest (minimum) value of all current limit inputs, such as drive peak, motor peak, and current limit (user limit). |

Repeat for each axis.

## Digital I/Os

The Inputs and Outputs panes enables you to configure the functionality and polarity of the digital inputs and outputs, and to monitor the state of all digital inputs and outputs.

ServoStudio+ displays the actual number of I/Os and brakes connected to the drive, as configured in the drive firmware.

The Brakes pane in the Digital I/Os screen enables you to activate brake functionality and to monitor the state of all brakes. Brakes are controlled by dedicated outputs as configured in the drive firmware.

- In ServoStudio+, select Configuration > Digital I/Os.

Figure ‎8-6. 	Digital I/Os

Set the modes for the defined inputs and outputs in your application.

Input and output mode definitions are specified in the drive firmware.

Digital input modes:

Idle.

Emergency stop.

PLS.  Positive limit switch.

NLS. Negative limit switch.

Home switch.

Digital output modes:

Idle.

Fault.

## Autotune

|  | Caution! Tuning is potentially dangerous. Before starting any tuning routines, make sure there are no persons or obstructions within the motion envelope. Also make sure the emergency stop button is within your reach. |
| --- | --- |

- In ServoStudio+, select Tuning > Autotune.

Figure ‎8-7. 	Autotuning

Make sure the displayed values in Autotune Settings are suitable for your application.

| Autotune Settings |  |
| --- | --- |
| Maximum Acceleration/ Deceleration | User defined maximum acceleration/deceleration value. |
| Motion Range | User defined number of motor revolutions. |
| Cruise Velocity | User defined speed. |

Click Start Autotune.

When Autotune is complete, options are displayed for testing controller performance.

Select Target Position, Cruise Velocity and Acceleration.

Press Move & Plot.

View the results on the graph.

Figure ‎8-8. 	Autotuning test results

If the results are satisfactory, press Save to Drive.

If the results are not satisfactory, use the Softer/Stiffer slider in the upper right corner of the screen to adjust the controller performance.

If necessary, restart the autotuning process.

When you are satisfied with the Autotuning test results, save the parameters to the drive.

You can now proceed to optimize drive parameters for your particular application.

## Control Loop

| Note | Control loop manipulation is intended for users with suitable training and expertise. The basic drive configuration does not require any changes in control loop parameters. |
| --- | --- |

The first time a new drive is connected to ServoStudio+, the software displays the drive’s factory-defined (default) values for the control loops.

- In ServoStudio+, select Configuration > Control Loops.

Review the Current Loop, Velocity Loop, and Position Loop tabs.

Make sure the displayed settings are suitable for your application.

Modify the values for the control loops as required by your application.

#### Position Loop

The smartPX has a linear, proportional (P) controller with feedforward.

Figure ‎8-9. 	Position control loop

#### Current Loop

The current control loop tuning is derived from the motor properties and the bus voltage.

Figure ‎8-10. 	Current control loop

There is no need to manipulate values in the Current Loop screen, unless instructed to do so by Technical Support.

#### Velocity Loop

The smartPX has two velocity controllers:

Pseudo derivative feedback and feedforward (PDFF) controller

The PDFF controller has an integral gain and a proportional gain, with the addition of feedforward.

When an application requires maximum responsiveness, less integral gain is required, and the PDFF feedforward value can be set to a higher value. When an application requires maximum low-frequency stiffness, the feedforward value is lowered, which allows much higher integral gain without inducing overshoot. This will also cause the system to be slower in responding to the command. A mid-range feedforward value is usually suitable for motion control applications.

Standard pole placement (PP) controller

The PP controller uses three parameters for tuning the control loop: bandwidth, gain and tracking factor.

Figure ‎8-11. 	Velocity control loop

## Upgrade Drive Firmware

- Download the firmware file (MAXX_3.0.x.x.ZIP) from the Servotronix website [LINK], or contact Technical Support.

In ServoStudio+, go to Settings > Firmware Update.

In the Load File field, select the new firmware ZIP file, and click Upgrade.

Figure ‎8-12. 	Firmware update

# Warnings and Faults

## Warnings

| Warning Code | Message | Description | Likely Cause/s | Action Required |
| --- | --- | --- | --- | --- |
| 0x00400402 | Velocity loop parameter changed. Vel.execute is required. | The vel.execute command is required after the value of a velocity loop parameter is modified | One or more velocity control loop parameters was changed | Set vel.execute<axis>=1 |
| 0x00400701 | Brake release taking too long | Brake release time exceeds the defined timeout value | Took too much time to release the brake. | Check brake timeout parameter (brake.release.timeout) |
| 0x00400f04 | Motor position feedback device reports an internal warning. | Warning from feedback device. The warning varies, depending on the specific feedback device. | Error status reported from feedback device. | Contact Technical Support. |
| 0x00400f05 | Motor position feedback device not responding on time warning | Problem receiving data from feedback device (sensAR) | Incorrect configuration of encoder type | Check feedback type configuration (mfb.type) Contact Technical Support. |
| 0x00401205 | Interpolation error. Check Feedback Position Resolution. | 10% of interpolation results are incorrect | Problem with feedback device | Check feedback device type Validate actual position value (pfb) |
| 0x00401603 | Position loop parameter changed. Pos.execute is required. | The pos.execute command is required after the value of a position loop parameter is modified | One or more position control loop parameters was changed | Set pos.execute<axis>=1 |
| 0x00401a13 | Default drive configuration | Cannot load parameters. Default configuration used. | New drive in use The parameter file (servo.dat) was deleted from the drive | Create/ load parameters file |
| 0x00401c02 | Phase A Current Sensor Out Of Range | Current sensor reported value out of range | Current sensor configuratioon not within range | Check current sensor configuration If the warning persists, contact Technical Support. |
| 0x00401c04 | Phase C Current Sensor Out Of Range | Current sensor reported value out of range | Current sensor configuratioon not within range | Check current sensor configuration If the warning persists, contact Technical Support |
| 0x00401f01 | Gainset parameters changed. Vargains.execute is required. | The vargains.execute command is required after the value of a gainset parameter is modified | Gainset configuration parameter was changed | Set vargains.execute<axis>=1 |
| 0x00401f02 | Could not find suitable Gainset | Gainset not found in the current variable gains configuration | Variable gains is active and current position is incompatible with configured gainset | Check variable gainset configuration Check axis position If required, disable variable gains function |
| 0x00401f03 | Gainset is not in use or cannot be activated | Suitable gainset not in use (configured gainset) or cannot be used (problem setting gainset) | Gainset configured with inuse set to zero | Check parameter vargains.active and vargains.inuse Check gainset configuration |
| 0x00402507 | Phase Find current command is insufficient | Actual current command is less than phase find current command, but not equal to zero | Ilim value is less than the phase find current set by the user (phasefind.icmd). | Reduce phase find current command Increase Ilim, if defined by the user |
| 0x00402508 | Phase Find auto is on – if axis is enabled, phase find will start | Warning after activation of Phase Find | Phasefind.auto is 1, and the axis has not been enabled yet after power-up | If Automatic Phase Find after drive power up is desired, no action is required. After the axis is enabled, phase find will start and the warning will disappear If Automatic Phase Find is not desired, disable it (phasefind.auto=0) |
| 0x00402701 | uwSwap was executed. Zeroing is required. | When uwSwap is active, zeroing is required. | Phase Swap was performed | Execute the zeroing process |
| 0x00402801 | Field-Weakening config failed | Field-weakening not compatible with motor parameters | Field-weakening is not possible for the defined motor parameters. | Make sure motor parameters (peak current, poles, kt, inductance, resistance) are all correct. |
| 0x00409804 | Motor position feedback device reports counter overflow | Warning from multiturn feedback device | Error status reported from feedback device. | Contact Technical Support |
| 0x00409808 | Motor position feedback device reports battery alarm | Warning from multiturn feedback device | Low supply battery voltage The battery does not exist | Check/ change battery feedback device If the warning persists, contact Technical Support |
| 0x0040d403 | Vact configuration changed. Vel.execute is required. | The vel.execute command is required after the value of a velocity loop parameter is modified | One or more velocity control loop parameters was changed | Set vel.execute<axis>=1 |
| 0x0040d405 | Cannot calculate MVE – no Vact signal | No velocity feedback signal (Vact) received for Model Velocity Error calculation | Velocity feedback is disabled. This is extremely unusual, unless velocity feedback was deliberately disabled. | Enable Velocity feedback, or change opmode such that it is enabled (for example, switch to position opmode). Make sure the internal opmode matches the opmode (that is, not in phase find/ homing/ other). |
| 0x0040d406 | Cannot calculate MVE – no Vobs signal | No Velocity feedback signal (Vobs) received for Model Velocity Error calculation | Velocity observer is disabled | Design and enable a velocity observer using the GUI. To validate, make sure that vact.Kalman.L.execute=1 after design. |
| 0x0040d407 | Collision detection mode is disabled | collision.response is not zero but collision.detect.mode is zero (not configured) | Collision.response is not 0, but collision.detect.mode is 0 | Enable collision detection; set collision.detect.mode to a non-zero value |
| 0x0040dc03 | Regen Resistor Overload Warning | Regen resistor has been activated excessively within the last 5 seconds. It exceeds the maximum average power dissipated. | The regen resistor is overloaded | Check regen resistor configuration Check appropriation regen resistor for the setup |
| 0x0040dd01 | IPM OT | The actual temperature value is higher than the minimum (warning) threshold. | The actual IPM temperature value is greater than minimum threshold (temp.ipm.flt.low) | Check device temperature Check threshold parameter Check actual temperature parameter (temp.ipm) |
| 0x0040dd03 | Motor OT | The actual temperature value is higher than the minimum (warning) threshold. | The motor temperature value is greater than minimum threshold (temp.motor.flt.low) | Check motor temperature Check threshold parameter Check actual temperature parameter (temp.motor) |
| 0x0040de01 | Ambient OT | The actual temperature value is higher than the minimum (warning) threshold. | Power/ ambient device temperature is too high | Check ambient temperature Check cooling device, if exists |
| 0x0040de03 | Device MPU OT | The actual temperature value is higher than the minimum (warning) threshold. | Device core temperature too high Incorrect threshold configuration Problem reading device sensor | Check device temperature Check threshold parameter Check actual temperature parameter (temp.cpu.mpu) |
| 0x0040de05 | Device CORE OT | The actual temperature value is higher than the minimum (warning) threshold. | Device core temperature too high Incorrect threshold configuration Problem reading device sensor | Check device temperature Check threshold parameter Check actual temperature parameter (temp.cpu.core) |
| 0x0040de07 | Device IVA OT | The actual temperature value is higher than the minimum (warning) threshold. | Device core temperature too high Incorrect threshold configuration Problem reading device sensor | Check device temperature Check threshold parameter Check actual temperature parameter (temp.cpu.iva) |
| 0x0040de09 | Device DSP OT | The actual temperature value is higher than the minimum (warning) threshold. | Device core temperature too high Incorrect threshold configuration Problem reading device sensor | Check device temperature Check threshold parameter Check actual temperature parameter (temp.cpu.dsp) |
| 0x0040de0b | Device GPU OT | The actual temperature value is higher than the minimum (warning) threshold. | Device core temperature too high Incorrect threshold configuration Problem reading device sensor | Check device temperature Check threshold parameter Check actual temperature parameter (temp.cpu.gpu) |
| 0x00541601 | Position loop design failed – MKT is zero | Motor constant parameter value is zero | Motor constant parameter is set to zero value | Configure motor constant parameter (motor.kt) |
| 0x00541a03 | IPM Warning | IPM fault was reported. Enable command required to release latch | IPM fault was reported | Set enable command, if possible |
| 0x00541a12 | 15ISO out of range | The actual 15ISO value is greater than the maximum threshold or lower than the minimum threshold. | Problem reading device power board | Power cycle the drive If the error persists, contact Technical Support. |

## Faults

| Fault Code | CAN Error Code | Disable Action | Message | Description | Likely Cause/s | Action Required |
| --- | --- | --- | --- | --- | --- | --- |
| 0x00040302 | 0x8381 | StopOnPath | High IQ Current Detected | Actual Iq current exceeds the defined threshold Iq current is greater than user current limit (curr.lim.user) + drives peak current (drive.ipeak)*0.2 for more than 1[ms] | Unstable control loop Not correct motor parameters, limits | Check limits and setup configuration Check control loops tuning |
| 0x00040401 | 0x8482 | StopOnPath | Velocity Error Deviation Exceeded | The difference between velocity command and actual velocity is greater than maximum velocity error | Incorrect motor parameters configuration Incorrect control gains Limit configuration too low | Check the drive limits (position, velocity, current) Check control gains |
| 0x00040d00 | 0x2310 | StopOnPath | Motor foldback value is less than threshold value | Motor fold current dropped below the motor fold current fault threshold | Incorrect motion profile for the application Control gains, loads for axis Incorrect motor parameters configuration | If excessive load during motion: reduce payload, change motion profile (acc/ dec) Check control gains, loads Check motor parameters Check brakes release, if exists Check foldback threshold parameter (motor.foldback.flt.thresh) |
| 0x00040e00 | 0x2311 | StopOnPath | Drive foldback value is less than threshold value | Drive fold current dropped below the drive fold current fault threshold | Excessive load, acceleration/ deceleration in the motion profile for the application Incorrect drive configuration parameters | If excessive load during motion: reduce payload, change motion profile Check control gains, loads Check drive parameters Check foldback threshold parameter (drive.foldback.flt.thresh) If the error persists, contact Technical Support |
| 0x00041202 | 0x8480 | StopOnPath | Generated Vcmd exceeded velocity limit | Generated velocity command exceeded velocity limit | Configuration parameter vlim is less than maximum than maximum velocity limit it motion controller. | Check velocity limit in the motion controller and drive. Limit must be equal in both or higher in the drive. |
| 0x00041203 | 0x8483 | StopOnPath | Generated Acmd exceeded acceleration limit | Generated acceleration command exceeded acceleration limit | Configuration parameter uint.acc.max is less than maximum acceleration limit in motion controller. | Check acceleration limit in the motion controller and drive. Limit must be equal in both or higher in the drive. |
| 0x00041204 | 0x8580 | StopOnPath | Excessive number of extrapolations | The number of extrapolations performed exceeds the maximum threshold | Drive did not receive a motion command from motion controller more than 10 times. | Verify the EtherCAT messages transmission rate Verify an EtherCAT connection |
| 0x00041602 | 0x8611 | StopOnPath | Excessive deviation in position error | The difference between the position command and the actual position exceeds the maximum position error | Control gains not acceptable for axis Incorrect motor parameters configuration Position error deviation threshold configuration | Check control gains, loads Check brakes release, if exists Check position error deviation threshold configuration (pos.err.max) |
| 0x00041a0c | 0x2386 | StopOnPath | Deadtime phase U fault | The delay time between high and low PWM signals is less than the deadtime value calculated by the FPGA. | Incorrect configuration/ functionality deadtime for phase U | Check the deadtime setting parameter (drive.deadtime.watchdog) If the error persists, contact Technical Support |
| 0x00041a0d | 0x2387 | StopOnPath | Deadtime phase V fault | The delay time between high and low PWM signals is less than the deadtime value calculated by the FPGA. | Incorrect configuration/ functionality deadtime for phase V | Check the deadtime setting parameter (drive.deadtime.watchdog) If the error persists, contact Technical Support |
| 0x00041a0e | 0x2388 | StopOnPath | Deadtime phase W fault | The delay time between high and low PWM signals is less than the deadtime value calculated by the FPGA. | Incorrect configuration/ functionality deadtime for phase W | Check the deadtime setting parameter (drive.deadtime.watchdog) If the error persists, contact Technical Support |
| 0x00041b01 | 0x7386 | StopOnPath | EtherCAT SYNC Jitter too high | SYNC signal jitter greater than the threshold | The actual SYNC jitter is greater than the EtherCAT SYNC0 jitter limit (fb.sync.jitter.limit). | Check cable/ connector EtherCAT Check SYNC0 jitter limit (fb.sync.jitter.limit) |
| 0x00041b02 | 0x818d | StopOnPath | Packet lost | Consecutive count for packet loss is greater than the maximum threshold | Issue in EtherCAT line communication | Check EtherCAT cable master/ slave |
| 0x00041b03 | 0xf080 | StopOnPath | Fieldbus - Drive is active but not in Operational | Drive in Enabled state but communication state changed from OP to PREOP/ SAFEOP | Communication is down but drive remains in Enabled state | Change motion controller communication state to OP |
| 0x00042102 | 0x7091 | StopOnPath | Emergency Stop Issued | Emergency Stop digital input is activated | Emergency Stop button is in active state | Release the E-stop button Change, if require, digital input mode |
| 0x00042602 | 0x0000 | StopOnPath | Hardware positive limit switch is open | Positive hardware limit switch was activated | Active state of input configured as positive limit switch | Change position of motor against the sensor Deactivate digital input Change, if require, digital input mode |
| 0x00042603 | 0x0000 | StopOnPath | Hardware negative limit switch is open | Negative hardware limit switch was activated | Active state of input configured as negative limit switch | Change position of motor against the sensor Deactivate digital input Change, if require, digital input mode |
| 0x0004d402 | 0x8481 | StopOnPath | Velocity over speed | Actual velocity exceeded 125% of maximum velocity on 4 consecutive samplings | Incorrect motor parameters configuration Incorrect control gains Limit configuration too low | Check the drive limits (position, velocity, current) Check control gains |
| 0x0004dc01 | 0x3120 | StopOnPath | Under Voltage Fault | The bus voltage is below the minimum value | Power supply loss, poor power supply Excessive load during motion | Check mains voltage If excessive load during motion: reduce payload, change motion profile (acc/ dec) If the error persists, contact Technical Support |
| 0x0004dd02 | 0x4080 | StopOnPath | IPM over-temperature | Actual temperature greater than the maximum threshold | The actual IPM temperature value is greater than the maximum threshold (temp.ipm.flt.high) | Check device temperature Check threshold parameter Check actual temperature parameter (temp.ipm) |
| 0x0004dd04 | 0x4410 | StopOnPath | Motor over-temperature | Actual temperature greater than the maximum threshold | The actual motor temperature value is greater than the maximum threshold (temp.motor.flt.high) | Check motor temperature Check threshold parameter Check actual temperature parameter (temp.motor) |
| 0x0004de02 | 0x4310 | StopOnPath | Ambient OT | The actual ambient temperature value is higher than the maximum threshold | Power/ ambient device temperature too high | Check ambient temperature Check cooling device, if exists |
| 0x0004de04 | 0x4081 | StopOnPath | Device MPU over-temperature | The actual temperature value is greater than maximum threshold | Device MPU temperature too high Incorrect threshold configuration Problem reading device sensor | Check device temperature Check threshold parameter Check cooling device, if exists Check actual temperature parameter (temp.cpu.mpu) |
| 0x0004de06 | 0x4081 | StopOnPath | Device CORE over-temperature | The actual temperature value is greater than the maximum threshold | Device core temperature too high Incorrect threshold configuration Problem reading device sensor | Check device temperature Check threshold parameter Check cooling device, if exists Check actual temperature parameter (temp.cpu.core) |
| 0x0004de08 | 0x4081 | StopOnPath | Device IVA over-temperature | The actual temperature value is greater than maximum threshold | Device IVA temperature too high Incorrect threshold configuration Problem reading device sensor | Check device temperature Check threshold parameter Check cooling device, if exists Check actual temperature parameter (temp.cpu.iva) |
| 0x0004de0a | 0x4081 | StopOnPath | Device DSP over-temperature | The actual temperature value is greater than the maximum threshold | Device DSP temperature too high Incorrect threshold configuration Problem reading device sensor | Check device temperature Check threshold parameter Check cooling device, if exists Check actual temperature parameter (temp.cpu.dsp) |
| 0x0004de0c | 0x4081 | StopOnPath | Device GPU over-temperature | The actual temperature value is greater than the maximum threshold | Device GPU temperature too high Incorrect threshold configuration Problem reading device sensor | Check device temperature Check threshold parameter Check cooling device, if exists Check actual temperature parameter (temp.cpu.gpu) |
| 0x000c0f01 | 0x73a0 | Dynamic Brake Time | Motor position feedback communication error: response timeout CRC error in response HAL failure. | Error message from feedback device. The fault is active when the status counter becomes equal to or greater than status error threshold. | Cable of feedback device disconnected Incorrect configuration of feedback type Feedback cable not shielded | Check feedback cable: connection, supply Check feedback type configuration (mfb.type) If the error persists, contact Technical Support. |
| 0x000c2401 | 0xff08 | Dynamic Brake Time | Runaway Detected | Uncontrolled motor motion | Incorrect motor parameters configuration Incorrect motor phases connection order | Check motor parameters Check motor phases connection |
| 0x000cd401 | 0x8484 | Dynamic Brake Time | Extreme Velocity Over Speed | Actual velocity exceeded 150% of maximum velocity on 4 consecutive samplings | Motion command from master greater than drive velocity limit Motor runaway due to opposite phase angle or motion in opposite direction, or mismatched UVW Controller is unstable and vlim is too small. | Check velocity limit parameter in drive (vlim) Check gains control loop Check motor phases, phase angle |
| 0x00140301 | 0x8380 | Disable | Unstable Current Loop | Actual current on any one of the phases exceeds 125% of drive’s peak current for 10 consecutive RT cycles | Incorrect motor parameters configuration Incorrect motor phases order Incorrect control gains | Check the motor parameters If the error persists, contact Technical Support |
| 0x00140303 | 0x2383 | Disable | Invalid Phase A Current Sensing Offset | Actual Ia current exceeds the defined threshold. Fault prevents zero current reading while PWM is active Phase A current exceeds 5% of drive peak current (drive.ipeak) | Unstable current control loop Incorrect motor/ drive parameters Short circuit motor phases | Check current control loop gains Check limits and setup configuration Check motor phases If the error persists, contact Technical Support |
| 0x00140304 | 0x2384 | Disable | Invalid Phase C Current Sensing Offset | Actual Ic current exceeds the defined threshold. Fault prevents zero current reading while PWM is active Phase C current exceeds 5% of drive peak current (drive.ipeak) | Unstable current control loop Incorrect motor/ drive parameters Short circuit motor phases | Check current control loop gains Check limits and setup configuration Check motor phases If the error persists, contact Technical Support |
| 0x00140700 | 0x7180 | Disable | Unexpected brake engage | The axis received brake engaged indication without request | Motors brake cable disconnected Motors brake cable short circuit | Check motors brake cable connection Check voltage brake measure Check motor |
| 0x00140702 | 0x7181 | Disable | Brake Status Overload Fault | The FPGA reports an overload | Short to ground Thermal shutdown Thermal swing brakes status fault | Check brakes power supply If brakes not supported, disable brakes configuration |
| 0x00140703 | 0x7182 | Disable | Brake status open load, short to battery, reverse polarity fault | The FPGA reports a fault in brakes status | Brakes configured but not supported Brakes cable not connected to brakes Brakes short circuit Reverse polarity | Check brakes power supply If brakes not supported, disable brakes configuration |
| 0x00140f03 | 0x73a1 | Disable | Motor position feedback device reports an internal fault. | Error message from feedback device. Message varies, depending on the specific feedback device. | Error status reported from feedback device. | Contact Technical Support |
| 0x00141201 | 0x6380 | Disable | Interpolation Design Failed | Error in interpolation | Problem with EtherCAT cycle time or configuration/ sync | Check interpolation configuration Check cycle time configuration If the error persists, contact Technical Support. |
| 0x00141a01 | 0x2214 | Disable | Axis Over Current | Short circuit on the motor connection; excessive overshoot in the current loop | Short circuit IPM Power device temperature too high | Check motor phases cable Check trace recording axis current Check brakes If the error persists, contact Technical Support |
| 0x00141a02 | 0x2218 | Disable | IPM Fault | FPGA reports IPM fault | Short circuit on the motor phases IMP under voltage fault IPM over temperature fault | Check phases of the motor connection Check STO connector Check IPM temperature If the error persists, contact Technical Support |
| 0x00141a06 | 0x2385 | Disable | Encoder Over Current | Short circuit on encoder supply; Overload supply | Encoder unit reports over-current | Check motor encoder cable Check motor encoder voltage supply If the error persists, contact Technical Support |
| 0x00141a07 | 0x8481 | Disable | Watchdog expired | FPGA reports watchdog | Drive firmware is overloaded | Contact Technical Support. |
| 0x00141a09 | 0x7090 | Disable | FPGA Version Mismatch | Current FPGA version incompatible with supported FPGA version | FPGA version of software application does not match actual FPGA version | Contact Technical Support |
| 0x00141a0b | 0x3181 | Disable | STO fault | FPGA reports STO error indication | The STO signal is not connected when drive enabled | Check STO connector If the error persists, contact Technical Support. |
| 0x00141a0f | 0x2389 | Disable | Regen Over Current | FPGA reports over current on regen resistor | Short circuit regen resistor | Check regen resistor If the error persists, contact Technical Support. |
| 0x00141a11 | 0x8802 | Disable | RT Overload | Realtime overload | Drive firmware is overloaded | Contact Technical Support. |
| 0x00141c01 | 0x2215 | Disable | Phase A Current Sensor Power Fail | Current sensor A is shut down | FPGA reports current sensor supply is down | Contact Technical Support |
| 0x00141c03 | 0x2217 | Disable | Phase C Current Sensor Power Fail | Current sensor C is shut down | FPGA reports current sensor supply is down | Contact Technical Support |
| 0x00142200 | 0x8180 | Disable | Drive Locked | Security code and key do not match | Eeproms were changed New drive | Contact Technical Support for security key. |
| 0x00142500 | 0x0000 | Disable | Zeroing process failed to calculate feedback noise limits | Velocity feedback value is too high/ low in idle state. Expects value to be close to zero. | Motor encoder is incorrectly defined Feedback is too noisy or the motor is moving | Check the motor parameters Make sure motor is not moved by outside disturbances. |
| 0x00142501 | 0x0000 | Disable | Zeroing process cannot perform sufficient movement | Motor did not settle after motion for more than 30 seconds | Zeroing current amplitude is too high Motor parameters are wrong (usually an encoder bit) | Reduce current Check the motor parameters |
| 0x00142502 | 0x0000 | Disable | Zeroing process cannot perform sufficient movement | Motor did not sufficiently move during zeroing process | Zeroing current amplitude is too low Motor parameters are wrong (usually number of poles or encoder bit) The motor is physically held in place | Increase current Validate motor parameters Remove physical hold |
| 0x00142503 | 0x0000 | Disable | Ilim current is zero | Actual limit current is less than zero | Incorrect configuration of parameter curr.lim.user | Check parameters curr.lim.user and curr.lim.act |
| 0x00142504 | 0x0000 | Disable | Phase Find process exceeded timeout | After motion, the motor did not settle within the user-defined duration | Configuration of parameters phasefind.settling.overtime or phasefind.standstill.velocity.threshold is too low Wake and Shake PI gains are not stable Phasefind current is too high | Change settling overtime or velocity threshold value Change PI gains (usually, reduce kp) Reduce current amplitude |
| 0x00142505 | 0x0000 | Disable | Phase Find process did not perform sufficient movement during validation | After finding Mphase, attempted motion failed | Phase find process did not find the right Mphase Motor parameters are wrong (usually number of poles or encoder bit) The motor is physically held in place | Repeat phase find process Validate motor parameters Remove physical obstacle |
| 0x00142506 | 0x0000 | Disable | Phase Find current command is zero | Cannot perform phase find if the current command is zero | User set the phase find current to zero | Set phase find current to a non-zero value |
| 0x00142509 | 0x0000 | Disable | Directional Phase Find failed to initialize Mphase – no movement detected | No motion was detected while attempting to initialize directional phase find | Phase find current too low Axis is physically held in place Incorrect motor parameters | Increase phase find current Remove physical hold on axis Validate motor parameters |
| 0x00142900 | 0x0000 | Disable | Timeout occurred while waiting for stopping profile to be completed | Timeout in homing process occurred motion segment not completed. | Trigger required in homing mode was not executed in time | Check trigger segment configuration (home switch, limit switches, index) Check homing motion profile configuration If the error persists, contact Technical Support |
| 0x00149801 | 0x73a6 | Disable | Motor position feedback device reports over speed | The shaft rotation exceeds the speed specificed in the Electrical Specifications for Multi-turn Signal. | Attempted to connect encoder cables while drive power on | Power cycle the drive with encoder cables plugged in If the error persists, contact Technical Support. |
| 0x00149803 | 0x73a4 | Disable | Motor Position feedback device reports counting error | When the shaft of Smart-Abs is rotated at 100[rpm] or more, an error is detected every 45º in mechanical angle. The error is automatically released at every 45º when the deviation of one revolution data is reduced to less than ±22.5º (typ.) When the shaft of Smart-Abs is rotated at less than 100 [rpm], an error is always detected. Any deviation of one revolution data is automatically returned to the normal value at detection of error. | Problem reading data from feedback device | Check configuration of feedback type Check encoder cable If the error persists, contact Technical Support |
| 0x00149805 | 0x73a8 | Disable | Motor position feedback device reports over heat | The temperature of the encoder substrate exceeds the overheating detection temperature | Encoder temperature is too hot The temperature threshold is to low | Check encoder temperature Check encoder temperature threshold |
| 0x00149806 | 0x73a9 | Disable | Motor Position feedback device reports multiturn error | The feedback device reports multiturn signal error during power-on. | User configured feedback type (mfb.type) does not match relay motor feedback | Check configuration of feedback type Check feedback cable |
| 0x00149807 | 0x73aa | Disable | Motor position feedback device reports battery error | Error message from feedback device. This fault is related to multiturn feedback device. | Low supply battery voltage The battery does not exist | Check/ change battery in feedback device If the error persists, contact Technical Support. |
| 0x0014d404 | 0x0000 | Disable | Collision detected | Collision MVED exceeded the collision threshold | Physical collision Collision detection threshold is too low (collision.MVED.thresh) | Remove obstacle, disable and reenable collision detection to clear fault Increase threshold, disable and reenable collision detection to clear fault |
| 0x0014dc02 | 0x3110 | Disable | Over Voltage Fault | The bus voltage exceeds the maximum threshold value | Incorrect motion profile for the application Load of axis too high | Record vbus during motion Change acceleration/ deceleration of motion profile Tune/ check control gains and load on axis If the error persists, contact Technical Support |
| 0x0014dc04 | 0x3199 | Disable | Regen resistor overload fault | Regen resistor has been activated excessively within the last 5 seconds. It exceeds the maximum average power dissipated. | Incorrect motion profile for the application Load of axis too high Incorrect use or configuration of regen resistor | Check regen resistor Check control gains, loads Check motion application; should be acceptable for the setup (axis) If the error persists, contact Technical Support |
| 0x04041a08 | 0x3183 | StopOnPath | AC Loss | Power supply was down for duration defined by hw.acloss.filter.time | Cable main power is disconnected Faulty power supply | Check main power Check parameter hw.acloss.filter.time |

## Inhibits

| Inhibit Code | CAN Error Code | Disable Action | Message | Description | Likely Cause/s | Action Required |
| --- | --- | --- | --- | --- | --- | --- |
| 0x00801a0a | 0x0000 | None | STO warning | FPGA reports STO error indication from the power board. | - STO connector is unplugged - No 24V supply to the STO connector | - Check the STO connector - Check the power supply |
| 0x0080dc05 | 0x0000 | None | Inrush open warning | The bus voltage has not reached inrush threshold. | Main power supply is disconnected | - Check the main power supply - Read Vbus value - Make sure inrush.open.thresh value meets specifications - If the error persists, contact Technical Support. |